;+ IDL PROCEDURE

;

; NAME          frscan_rpb_ne.pro

;

; PURPOSE       Perform a Mk4 rpb radial scan using a FITS image [cartesian].

;               Compute the electron density from the pB intensity.

;               Generates radial scan plots for pB [Bsun] and Ne/cm2.

;

; SYNTAX        frscan_rpb_ne, fits_file, angle, radmin, radmax, radinc

;

; PARAMETERS    fits_file       name of FITS file.

;               angle           position angle [degrees CCW from solar North]

;               radmin          beginning radius [Rsun]

;               radmax          ending    radius [Rsun]

;               radinc          angle increment  [Rsun]

; OPTIONAL

;               ymin            Y-axis minimum value

;               ymax            Y-axis maximum value

;

; OUTPUTS       GIF file showing the position of the radial scan.

;               POSTSCRIPT file containing the radial scan plots.

;               TEXT file containing the scan values [ASCII].

;

; EXT.ROUTINES  rscan.pro       Performs radial scan.

;               lct.pro         loads a color table from an ASCII file.

;               rcoord.pro      converts between [r,th] and [x,y] coordinates.

;               nedpb.pro       computes Ne/pB[Bsun]-cm2 for a specified radius.;

; NOTE          This procedure works best in private colormap mode (256 levels).;

; AUTHOR        Andrew L. Stanger   HAO/NCAR   18 Jun 2004

;-