CHIP Calibration using 7 filter positions.		 8 March 2001

1. Read in raw chip FITS images taken at 7 different filter positions.

index	filter	wsr		filter

value	name	designation	description

-----	------	-----------	-----------

1	B3	dsr		far blue

2	B2	csr		middle blue

3	B1	bsr		near blue

4	He	hsr		on-line Helium

5	R1	rsr		near red

6	R2	ssr		middle red

7	R3	tsr		far red

2. Extract relevant information from FITS header:

xdim, ydim

bitpix (-> nbypix)

pixrs

rsun

P-angle

WSR code (wavelength-source-reduction)

wavelength

3. Remove binary header.

4. Determine disk center.

A. Read crpix1/crpix2 values from FITS header.

B. If crpix1/crpix2 are invalid (negative), then

use Randy Meisner's initgeom software to compute disc center.

C. If this fails, assume disk center is at array center.

4. Crop images to 936 x 936 square (with disc center at array center).

5. Rotate images by P-angle, thereby converting to Heliocentric coordinates.

6. Read image distance file [e.g., 20010105.180145.chp.hsr.rad.fts].

[Create radial distance file using C-language program: chip_radius_fits.c]

7. Read doppler correction file (B3 only).

8. Read limb darkening fit files ["ldf" files created using IDL ld3.pro].

An ldf file is previously created for each wavelength.

Compute limb darkening correction:

imgldc = imgmax / imgldf

9. Apply limb darkening correction.

[Use limb darkening derived from dsr image to correct dsr image.]

[Use limb darkening derived from ssr image to correct all other images.]

Multiply each image by the corresponding limb darkening correction image.

10. Compute the average of the brightest 5% of the pixels on the solar disk.

imax1 ... imax7.

11. Compute continuum brightness:

Bcont = imax2 / 0.97

12. Find the brightest 20% pixels within an equatorial "band" in the

Helium on-band image.

13. Extract a band from each image, and compute the average of the

20% brightest pixel locations found in the previous step.

14. Compute normalization correction terms:

nc1 = Bcont - brtavg1

nc2 = Bcont - brtavg2

nc6 = Bcont - brtavg6

nc7 = Bcont - brtavg7

nc3 = 0.8 * nc2

nc4 = 0.4 * nc2 + 0.04 * nc6

nc5 = 0.2 * nc2 + 0.6  * nc6

NOTE: currently, nc3, nc4 & nc5 are computed as follows:

nc3 = Bcont - brtavg3

nc4 = Bcont - brtavg4

nc5 = Bcont - brtavg5

15. Normalize brightess by addition correction terms.

img1n = img1f + nc1

img2n = img2f + nc2

img3n = img3f + nc3

img4n = img4f + nc4

img5n = img5f + nc5

img6n = img6f + nc6

img7n = img7f + nc7

16. Compute Helium-I image:

imghe1 = img4n - bcont

17. Compute average of all 7 normalized images:

imgavg = (img1n + img2n + img3n + img4n + img5n + img6n + img7n) / 7.0

18. Compute velocity image:

imgdif0 = 1.0 - ( imgavg / bcont )

imgdif1 = (img3n - img5n) / bcont

imgdif2 = (img2n - img6n) / bcont

imgdif3 = (img1n - img7n) / bcont

imgvel =   (  4.0 / ( imgdif0 ) )

* (        ( imgdif1 )

+ 2.0 * ( imgdif2 )

+ 3.5 * ( imgdif3 )

+ 6.0 * ( imgrem) ^ 1.4 )

Set imgvel to zero if absolute value of imgdif0 is greater than 0.01