* * * * * * * * * * Mark-III K-coronameter Synoptic Maps * * * * * * * * * * *

20 January 1994

Andrew L. Stanger

1. As user "cordyn" on puu, use mkamap to create binary synoptic data files.

For example, I have used 1.25, 1.50, 1.75, and 2.00 Rsun as

four different heights at which to initially generate synoptic maps.

Typing

mkamap -l -m -e -h 1.25 -r 1875 `cat pb.ls`

will generate the East limb synoptic data file for C.R.1875 @1.25 Rsun.

The file "pb.ls" contains a list of the ".pB" files.  Thus far, I

have simply created this by typing "ls *.pB > pb.ls" while situated

in directory /puu/ftpmk3/mlsodat/mk3_best (same as ~ftp/mk3/mlsodat/mk3_best

when logged on to puu).

Generate data files for both east and west limbs.

2. The synoptic map annotation program is called "mk3syn" and is

located in "/riker/usr/people/stanger/mlso/mk3/syn".  It must

be executed from an SGI workstation (Picard or Riker).

For instance, typing:

/riker/usr/people/stanger/mlso/mk3/syn/mk3syn 1875e1.25 1875w1.25

will produce a file with the name "1875.r125.map", which is a

512x512x8-bit file, suitable for viewing on the Grinnell.

3. You may use either Picard or Riker with WHIP to display such a

synoptic map in a frame buffer (use /riker/usr/people/stanger/fb/grnv)

. 0				set FB #0

seelo			or type "see -u 1 760 480 1271 991"

grnv 1875.r125.map		display file in FB #0.

Or you may display it on an SGI using /riker/usr/people/stanger/gl/img/grnw).

Or you may display it on the Grinnell using grndisp.

4. The next step is to create a Sun rasterfile of the synoptic map.

Thus far, I have used color table "art2" on riker.  This may be

loaded by typing "clut /riker/usr/people/stanger/color/art2".

This LUT has yellow near the top of the colormap.  An alternative

color table (without the yellow) is "art3", which might be

preferable if the maps are to be eventually printed in B/W.

The reason is that yellow shows up as darker than white when

a B/W print is generated using Postscript.

A greyscale colortable may be loaded by typing:

clut /riker/usr/people/stanger/color/greycp

Or a blue-white-yellow color table is loaded by typing:

clut /riker/usr/people/stanger/color/bwy

Or a blue-white color table is loaded by typing:

clut /riker/usr/people/stanger/color/blue_white_cp

Next, the image & current color LUT are saved by typing:

/riker/usr/people/stanger/fbsrf 1875.r125.map.sun

to create a sun rasterfile from the image stored in the current

frame buffer (by default, FB#0).

Or you could do this step on the Grinnell using grnsrf.

5. Now you need to convert this sun rasterfile to postscript.

You probably have a method to do this using Framemaker.

I have used a somewhat lengthy procedure of converting the

sun rasterfile to SGI format (uses the extension .rgb) with

imconv, then using Showcase to read the .rgb file in and

then saving it as a postscript file.

A simplier way to do this (on a sun workstation) is to use "rash":

rash -W 7.11in 1875.r125.map.sun > 1875.r125.map.ps

Or on an SGI workstation, type:

imconv 1875.r125.map.sun 1875.r125.map.rgb

tops 1875.r125.map.rgb -p 72 -o 0.75 -1.0 -RGB > 1875.r125.map.ps

The postscript file may be sent to the HAO color printer by typing:

lpr -Phaocolor 1875.r125.map.ps

I know this is a fairly lengthy procedure, but it does work.

Obviously, some effort needs to be done to streamline it.

IDL could be used instead, but it's a bit tricky to maintain

consistent results due to the variable number of available

color LUT levels, which varies depending on the workstation and

window manager being used.

I've put C.R. 1875 maps in the puu FTP directory /puu/ftpmk3/map.

You'll find the raw data files (from mkamap) in sub-directory "raw",

the Sun rasterfiles in "srf", and the postscript files in "ps".

* * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *