CHIP Limb Darkening correction.					3 Oct 2000

chip_ldark11.c

--------------

Input raw CHIP images:

dsr	far  blue

csr	mid  blue

bsr	near blue

hsr	Helium on-band

rsr	near  red

ssr	mid   red

tsr	far   red

Read binary header.

Compute average intensity at sun center [10x10 pixel box].

For B3 (far blue) filter, compute "doppler" correction:

1.5% peak-to-peak linear variation from east to west limb.

Apply doppler correction to B3 filter only.

Perform radial scans: 90 & 270 degrees (heliocentric coordinates).

Average together the two radial scans.

For each pixel:

Determine radius.

Use radius to get intensity from average radial scan.

Divide average central intensity by the radial profile itensity

to determine the limb-darkening correction.

Apply limb darkening correction to pixel.